Class ControlBCP8000

Class ControlBCP8000 serves to download and transfer data saved in the BCP - 8000 data collector to a text string. The development works under Visual Studio.Net 2005. Please go to Microsoft’s official website to download and install Dot Net Frame Work 2.0.

Properties

| PortName | Read/Write | String |
| --- | --- | --- |
| Set the name of the COM Port for communication, which can be “COM1”, “COM2”…etc. |  |  |
| BaudRate | Read/Write | Integer |
| Set the baud rate of the COM Port for communication, which can be “9600”, “19200”, “38400”, “57600”, ”115200” Default value is 115200. |  |  |

Methods

Boolean AutoPowerOff(String mPassword, Integer iMinutes)

To set auto power off postpone time of BCP-8000 data collector.
mPassword is a password string (up to 10 digits) for security checking. iMinutes is auto power off postpone time in minutes. If succeeds, it will report True; otherwise False will be reported.

Boolean BuzzerHz(String mPassword, Integer iHz)

To set BCP-8000 data collector buzzer’s frequency.
mPassword is a password string (up to 10 digits) for security checking. iHz is the frenquency of buzzer. If succeeds, it will report True; otherwise False will be reported.

Boolean DownloadLookup(String mPassword, String mFile)

Download the destined lookup file to BCP-8000 data collector.
mPassword is a password string (up to 10 digits) for security checking. mFile is your lookup file’s full file name conbined path. If succeeds, it will report True; otherwise False will be reported.

Boolean DownloadProgram(String mPassword, String mFile)

Download the current application template to BCP-8000 data collector.
mPassword is a password string (up to 10 digits) for security checking. mFile is your application’s full file name conbined path. If succeeds, it will report True; otherwise False will be reported.

Boolean ChangePassword(String mPassword, String mNewPassword)

Change the password of the BCP unit. 
mPassword is current password of BCP unit and mNewPassword is the new password you want to change to. If succeeds, it will report True; otherwise False will be reported..

Boolean DownloadRTC(String mPassword)

Correct the internal clock of the BCP unit to synchronize with the computer’s clock. This method retrieves the date and time of the computer and then sends to the BCP unit.
mPassword is a password string (up to 10 digits) for security checking. If succeeds, it will report True; otherwise False will be reported.

Boolean LCDBackLight(String mPassword, Integer iSeconds)

To turn on LCD backlight in an assigned time interval for BCP-8000 data collector.
mPassword is a password string (up to 10 digits) for security checking. iSeconds is seconds of backlight on. The LCD always turn on when iSeconds = 255. If succeeds, it will report True; otherwise False will be reported.

Boolean Prompting (String mPassword, Integer RecordPrompting)

To adjust the time of  showing record counts while finishing each record of BCP-8000 form.
mPassword is a password string (up to 10 digits) for security checking. RecordPrompting is the time interval of showing record counts (in milliseconds).

0 <= RecordPrompting <= 9999.

If succeeds, it will report True; otherwise False will be reported.

Boolean TestBuzzer (String mPassword, Integer iHz)

To beep buzzer once with the designated frequency (not set).
mPassword is a password string (up to 10 digits) for security checking. iHz is the frenquency of buzzer. If succeeds, it will report True; otherwise False will be reported.

Boolean ReceiveData(String mPassword, Byte ArrayX[])

Download data saved in the BCP - 8000 data collector to a binary byte array.
mPassword is a password string (up to 10 digits) for security checking.

If succeeds, it will report True; otherwise False will be reported.

Boolean UpdateKernel(String mPassword, String mFile)

Update new version kernel of the BCP-8000. mPassword is a password string (up to 10 digits) for security checking. mFile is your configuration’s full file name conbined path. If succeeds, it will report True; otherwise False will be reported.

String KernelVersion(String mPassword)

To get kernel version from BCP - 8000 data collector.
mPassword is a password string (up to 10 digits) for security checking.